WESAD Stress Detection

Complete Technical Documentation

Binary Stress Detection Using Wearable Physiological Signals

Kaustubh Agrawal

April 8, 2026

# 1. Executive Summary

This project builds a complete stress detection system — from raw physiological signals to edge-deployable models — using the WESAD dataset (15 subjects, chest-worn RespiBAN sensors at 700 Hz).

Phase 1 (Traditional ML): Load raw pickles → Butterworth filtering → z-score normalization → 60-second sliding windows → ~150 hand-crafted features → Logistic Regression and Random Forest → LOSO cross-validation. Result: 0.96+ accuracy, 0.95+ recall.

Phase 2 (Deep Learning + KD): Train a multi-scale 1D-CNN teacher (266K params) → distill into three student architectures → LOSO validation. Result: MicroCNN (distilled, ~5K params) achieves 0.998 accuracy and 0.996 F1 — matching the teacher at 50x compression.

The project includes SHAP-based channel importance analysis, Grad-CAM temporal activation maps, KD hyperparameter ablation studies, and INT8 quantization benchmarks.

# 2. Problem Statement & Motivation

Chronic stress affects ~77% of adults and contributes to cardiovascular disease, mental health disorders, and reduced quality of life. Most individuals lack objective, continuous tools to monitor stress. This project asks: Can we build a model that reliably detects stress from wearable physiological signals, generalizes to unseen users, and runs on resource-constrained edge devices?

## Hypotheses

H1: Physiological signals contain measurable patterns that reliably distinguish stress from baseline states.

H2: Traditional ML with hand-crafted features can achieve >70% recall under LOSO validation.

H3: Knowledge distillation can compress a large teacher CNN into microcontroller-sized models without significant performance loss.

All three hypotheses were validated. H2 was exceeded by a wide margin (recall >95%).

# 3. Dataset Description

| Attribute | Details |
| --- | --- |
| Source | TU Darmstadt (Schmidt et al., 2018) |
| Subjects | 15 participants (S2–S17, S12 excluded) |
| Duration | ~2 hours per subject |
| Device | RespiBAN Professional (chest-worn) |
| Sampling Rate | 700 Hz |
| File Format | Python pickle (.pkl), ~975 MB each |
| Total Size | ~15 GB |

## Signals Used

| Signal | Description | Stress Relevance |
| --- | --- | --- |
| ECG | Electrocardiogram | Heart rate variability, cardiac stress |
| EDA | Electrodermal Activity | Skin conductance, sympathetic arousal |
| EMG | Electromyography | Muscle tension |
| Resp | Respiration | Breathing rate and depth changes |
| Temp | Body Temperature | Thermoregulation under stress |
| ACC | 3-axis Accelerometer | Motion artifact detection |

## Binary Classification Task

Labels are remapped: baseline (Label 1) → 0, stress (Label 2) → 1. All other labels are discarded. This creates a ~64/36 class split (baseline-heavy), handled via balanced class weights and weighted sampling.

# 4. System Architecture

The pipeline has two phases sharing common data loading, preprocessing, and windowing stages:

Phase 1 path: windows → hand-crafted features (~150) → LogReg / RandomForest → LOSO evaluation.

Phase 2 path: windows → FFT downsample to 64 Hz → Teacher CNN training → Student CNN training (standalone + distilled) → LOSO evaluation.

## Key Configuration

| Parameter | Value | Purpose |
| --- | --- | --- |
| CHEST_SAMPLING_RATE | 700 Hz | Raw data rate |
| WINDOW_LENGTH_SEC | 60 s | Window duration |
| WINDOW_OVERLAP | 0.5 | 50% overlap |
| PURITY_THRESHOLD | 0.8 | Min label purity per window |
| TRANSITION_BUFFER_SEC | 5 s | Discard zone around transitions |
| DL downsample target | 64 Hz | DL input rate |
| Teacher epochs | 40 | Training epochs |
| Student epochs | 30 | Training epochs |
| KD temperature | 4.0 | Softmax temperature |
| KD alpha | 0.7 | Teacher weight in KD loss |
| Learning rate | 3e-4 | AdamW optimizer |
| Random seed | 42 | Reproducibility |

# 5. Phase 1: Traditional ML Pipeline

## 5.1 Signal Preprocessing

Each signal gets a modality-specific 4th-order Butterworth filter (zero-phase via filtfilt):

| Signal | Filter Type | Cutoff | Rationale |
| --- | --- | --- | --- |
| ECG | Bandpass | 0.5–40 Hz | Remove baseline wander + HF noise |
| EDA | Lowpass | <1 Hz | EDA is slow-varying |
| EMG | Bandpass | 20–300 Hz | Muscle activity range |
| Resp | Bandpass | 0.1–0.5 Hz | Normal breathing frequency |
| Temp | Lowpass | <0.1 Hz | Very slow thermal changes |

Normalization: Z-score per subject (z = (x - mean) / std). Robust z-score (median/IQR) for EDA and Temp. NaN values replaced with per-signal median.

## 5.2 Windowing & Segmentation

60-second windows (42,000 samples at 700 Hz) with 50% overlap (21,000-sample step). Quality filters: 80% purity threshold (min label agreement within window) and 5-second transition buffer (samples near label changes are excluded).

## 5.3 Feature Engineering

~150 hand-crafted features per window across four categories:

| Category | Examples | Count |
| --- | --- | --- |
| Statistical | Mean, std, variance, min, max, range, percentiles, IQR, skewness, kurtosis, RMS, energy | ~10/signal |
| Temporal | Zero-crossing rate, peak count, slope features | ~5/signal |
| Frequency | Welch PSD, power bands (VLF/LF/HF/VHF), peak freq, spectral centroid/entropy | ~5/signal |
| EDA-Specific | Tonic/phasic decomposition, SCR count, SCR amplitude | ~5 total |

## 5.4 Models

Logistic Regression: L2 penalty, C=1.0, balanced class weights, lbfgs solver, max_iter=1000.

Random Forest: 100 trees, max_depth=10, min_samples_split=5, balanced class weights.

Both use StandardScaler (fit on training data only).

## 5.5 Baselines

Random: Samples from class prior. Recall = 0.358, F1 = 0.364.

Majority: Always predicts baseline. Recall = 0.000, F1 = 0.000.

EDA Threshold: Per-subject threshold on mean EDA. Recall = 0.866, F1 = 0.792.

# 6. Phase 2: Deep Learning & Knowledge Distillation

## 6.1 Data Preparation

700 Hz signals are downsampled to 64 Hz via FFT-based resampling. Input tensor: (batch, 6, 3840) — 6 channels x 60s x 64 Hz. NaN sanitization applied before/after resampling with ±5 clipping.

## 6.2 Teacher: MultiScaleTeacherCNN (~266K params)

Three parallel convolutional branches at different kernel sizes capture physiological phenomena at different timescales:

| Branch | Kernel | Captures | Physiological Target |
| --- | --- | --- | --- |
| Small | k=8 | Fast patterns | ECG heartbeats, motion |
| Medium | k=32 | Mid-range | Respiration cycles |
| Large | k=64 | Slow trends | EDA rise, thermal drift |

Each branch: Conv1d → BN → ReLU → Conv1d → BN → ReLU → GlobalAvgPool. Outputs concatenated (192-dim) → FC(192→128) → ReLU → Dropout(0.3) → FC(128→64) → ReLU → FC(64→2).

## 6.3 Student Architectures

| Model | Params | Architecture | Target Device |
| --- | --- | --- | --- |
| MicroCNN | ~5.3K | Depthwise-separable Conv1d (MobileNet-style) | ARM Cortex-M4 (<32KB) |
| TinyCNN | ~15.2K | 3-layer standard Conv1d | nRF52840 (<256KB) |
| MiniCNN-LSTM | ~28.8K | Conv1d + LSTM hybrid | ESP32-S3 (<512KB) |

## 6.4 Knowledge Distillation

KD loss (Hinton et al., 2015): L_KD = α × T² × KL(softmax(s/T) || softmax(t/T)) + (1-α) × CE(s, y_true)

T=4.0 softens teacher output, exposing inter-class similarities. α=0.7 gives 70% weight to soft targets. T² scaling restores gradient magnitude.

Training: AdamW (lr=3e-4, wd=1e-3), ReduceLROnPlateau (F1, patience=5), gradient clipping (max_norm=1.0), WeightedRandomSampler for batch-level class balance.

## 6.5 LOSO Evaluation

All models evaluated with Leave-One-Subject-Out: train on 14 subjects, test on 1, repeat for all 15. Report mean ± std across folds. This simulates real-world deployment on unseen users.

# 7. Explainability & Interpretability

## 7.1 SHAP Channel Importance

ECG dominates with mean |SHAP| = 0.00093, followed by Temp (0.00029), EDA (0.00025), ACC (0.00023), EMG (0.00016). Respiration contributes near-zero.

Figure 8: Teacher CNN — Mean |SHAP| per Signal Channel (Stress Class)

## 7.2 Per-Class SHAP

ECG matters ~1.75x more for stress windows (0.00128) than baseline (0.00073). EDA shows 2.4x higher importance for stress. Temp and ACC are more important for classifying baseline — the model uses temperature/motion stability as a baseline indicator.

Figure 10: Per-Class SHAP — Stress vs Baseline Windows

## 7.3 Grad-CAM Temporal Activation

The teacher's fast branch (k=8) shows consistently higher activation for stress (~0.33) vs baseline (~0.17) across the entire 60-second window. Activation is uniform temporally — the model uses the full window.

Figure 9: Grad-CAM Temporal Activation — Stress vs Baseline

## 7.4 Knowledge Distillation — Preserved Attention

After 50x compression (266K → 5.3K), the MicroCNN student preserves general attention patterns but redistributes: less ECG reliance (0.00012 vs 0.00093), more Temp (0.00047 vs 0.00028) and EDA (0.00035 vs 0.00024).

Figure 11: Teacher vs Student SHAP Channel Importance

# 8. Results & Analysis

## 8.1 Complete Results

Figure 3: Complete Results Summary — All Models (LOSO)

Figure 1: Model Comparison — Recall, F1, ROC-AUC (LOSO)

## 8.2 Key Findings

Finding 1 — Traditional ML is surprisingly strong. LogReg and RF achieve 0.95+ recall with hand-crafted features, far exceeding the 70% target.

Finding 2 — Multi-scale teacher improves consistency. Teacher CNN (0.981 F1) beats RF (0.956 F1) with lower variance across subjects.

Finding 3 — KD produces the best model overall. MicroCNN (distilled, 5K params) achieves 0.996 F1 — surpassing its own teacher at 50x compression.

Finding 4 — Distillation helps small models most. MicroCNN gains +0.039 F1; MiniCNN-LSTM gains +0.014; TinyCNN shows -0.004 (negligible).

Figure 5: Knowledge Distillation Improvement over Standalone Training

## 8.3 Per-Subject Analysis

S2 (LogReg F1=0.74), S3 (LogReg F1=0.64), S8 (RF F1=0.69) are problematic subjects. Deep learning models largely eliminate these per-subject weaknesses.

Figure 6: Per-Subject LOSO F1 Heatmap — All Models

Figure 2: LOSO Per-Subject Performance (LogReg vs RF)

## 8.4 Pareto Front

Distilled students dominate the upper-left corner: high F1 (>0.99) at minimal model size (<60KB). The teacher sits at ~1MB — overkill for deployment.

Figure 4: Accuracy vs Model Size — Pareto Front

## 8.5 KD Ablation

F1 converges to 0.9965 across all tested temperatures (1, 2, 4, 8) and alphas (0.3, 0.5, 0.7, 0.9). The task has high signal-to-noise ratio and the student has sufficient capacity.

Figure 7: KD Hyperparameter Ablation — Temperature and Alpha

## 8.6 Hypothesis Validation

| Hypothesis | Verdict | Evidence |
| --- | --- | --- |
| H1: Physio signals distinguish stress | Supported | ROC-AUC 0.976–1.000 |
| H2: ML achieves >70% recall (LOSO) | Strongly Supported | All models exceed 0.95 recall |
| H3: KD compresses without loss | Supported | MicroCNN (5K) surpasses teacher (266K) |

# 9. Design Decisions & Rationale

60-Second Windows: Captures full EDA stress response (latency 1–3s, full response 10–15s). 30s risks truncation; 120s reduces sample count.

50% Overlap: Balances sample augmentation vs window independence. 0% = too few samples; 75% = too correlated.

80% Purity: Ensures unambiguous physiological state. 100% is too strict; 60% allows mixed states.

Per-Subject Z-Score: Resting heart rate varies 40–100 BPM across individuals. Per-subject normalization removes this.

Multi-Scale CNN: Parallel branches (k=8/32/64) capture ECG (~1 Hz), respiration (~0.2 Hz), and EDA (~0.05 Hz) timescales.

Depthwise-Separable Conv: ~8x parameter reduction vs standard Conv1d. Enables MicroCNN at ~5K params.

Response-Based KD: Architecture-agnostic, simpler than feature-based KD, sufficient for binary task with high SNR.

Double Imbalance Defense: WeightedRandomSampler + balanced class weights. Without both, recall collapsed to 0 in early experiments.

# 10. Known Limitations & Open Questions

Controlled lab setting (TSST) may not generalize to everyday stress. 15 subjects cannot capture population diversity. Chest-only signals are impractical for daily use. Binary classification only. No real-time streaming. Some respiration NaN values of unclear origin.

## Open Questions

| ID | Question | Priority |
| --- | --- | --- |
| 1 | Optimal filter cutoffs for stress detection? | Medium |
| 2 | Respiration NaN root cause? | High |
| 3 | Feature redundancy (correlation analysis)? | Medium |
| 4 | Why do S2, S3, S8 underperform? | High |
| 5 | Cross-dataset validation (SWELL, AMIGOS)? | High |

# 11. Deployment Considerations

| Model | Params | FP32 Size | Target Device | Flash Budget |
| --- | --- | --- | --- | --- |
| MicroCNN (distilled) | ~5.3K | ~21 KB | ARM Cortex-M4 | <32 KB |
| TinyCNN (distilled) | ~15.2K | ~60 KB | nRF52840 | <256 KB |
| MiniCNN-LSTM (distilled) | ~28.8K | ~113 KB | ESP32-S3 | <512 KB |

INT8 dynamic quantization achieves 3–4x further compression. MicroCNN at INT8 is ~6 KB.

Inference input: 60s of 6-channel data at 64 Hz = 23,040 floats (~92 KB FP32, ~23 KB INT8).

# 12. How to Reproduce

Phase 1: python main.py

Teacher: python train_teacher.py

Students: python train_students.py

Ablation: python run_ablation.py

Figures: python generate_report.py

SHAP: python shap_analysis.py

Dataset: WESAD (not redistributable). Request from Schmidt et al., 2018, TU Darmstadt.

# 13. References

1. Schmidt, P., Reiss, A., Duerichen, R., Marber, C., & Van Laerhoven, K. (2018). Introducing WESAD, a multimodal dataset for wearable stress and affect detection. ICMI 2018.

2. Kirschbaum, C., Pirke, K. M., & Hellhammer, D. H. (1993). The Trier Social Stress Test. Neuropsychobiology.

3. Hinton, G., Vinyals, O., & Dean, J. (2015). Distilling the knowledge in a neural network. arXiv:1503.02531.

4. Howard, A. G., et al. (2017). MobileNets: Efficient convolutional neural networks. arXiv:1704.04861.

5. Lundberg, S. M., & Lee, S. I. (2017). A unified approach to interpreting model predictions. NeurIPS 2017.